Connecticut  Coastal

ADS-40 Imagery Collection Project

- ADS40 Imagery Collection Project

- Connecticut Coastal

- EarthData International of Maryland, Inc requested the collection of ADS-40 imagery data over Coastal Connecticut.  The data was collected to support the generation of natural color and color infrared ortho-photographs with a 0.50 meter ground sample distance (GSD).  In response EarthData Aviation acquired image data on September 20th and September 22nd, 2004.  The collection was performed using Horizons’ Cessna Conquest with tail number N93HC.  Image data was captured using Leica ADS40 camera systems, including an inertial measuring unit (IMU) and a dual frequency GPS receiver.

GPS Data Collection

During airborne data collection, a network of ground GPS receivers was in constant operation during data collection. All stations used were tied to the National Geodetic Survey (NGS) Continuously Operating Reference Station (CORS) network. For each lift, the flight crew set up a 1 second GPS receiver on an NGS published point. The CORS stations served as secondary receivers to ensure adequate coverage during flight.

Base station identifiers and their corresponding coordinates are listed in the table below, along with the lift numbers in which they were used as the primary base station. CORS stations and NGS published points are listed by the NGS identifiers, while temporary points use field crew nomenclature.

| Lift Number | Base Station Name | Base Latitude (N) | Base longitude (W) | Ellipsoidal Height (m) |
| --- | --- | --- | --- | --- |
|  |  | (ddd-mm-ss.sss) | (ddd-mm-ss.sss) |  |
| 26404B | A15582 | N 41 16 04.79476 | W 072 53 18.39305 | -28.56 |
| 26604A | DF8964 | N 40 47 03.55012 | W 073 05 49.78136 | 7.65 |

GPS Data Processing

All GPS phase data was post processed with continuous kinematic survey techniques using “On the Fly” (OTF) integer ambiguity resolution.  The GPS data was processed with forward and reverse processing algorithms.  The results from each process, using the data collected at the airport, were combined to yield a single fixed integer phase differential solution of the aircraft trajectory.  Plots of altitude and the forward and reverse GPS solution residuals are attached.  The difference between the forward to reverse solutions for each lift indicate valid and accurate solutions.  These differences are summarized in the table below:

| Lift | Date | Forward – Reverse Separations (meters) |  |
| --- | --- | --- | --- |
|  |  | Horizontal Component | Vertical Component |
| 26404B | 09/20/2004 | .06 | .17 |
| 26604A | 09/22/2004 | .06 | .22 |

Orientation Data Processing

An IMU was used to record precise changes in position and orientation of the ADS-40 camera at a rate of 200 Hz.  All IMU data was processed post flight with a Kalman filter to integrate inertial measurements and precise phase differential GPS positions.  The resulting solution contains geodetic position, omega, phi, kappa, and time for subsequent merging with the laser ranging information.

- ADS-40 Data Collection

The areas of interest were flown at an altitude of 15,750 feet above mean terrain.  The ADS-40 collection specifications follow:

| Sensor Collection Parameters |  |
| --- | --- |
| Flying Height | 15,750 feet AMT |
| Target Airspeed | 270 knots |
| Desired GSD | 0.50 meters |
| Camera Cycle Time | 3.6 ms |
| Collection Mode | Standard All |

The table below summarizes the data collection by date:

| Data Collection By Date |  |  |
| --- | --- | --- |
| Lift | Date | Lines Collected |
| 26404B | 09/20/2004 | 2 - 17 |
| 26604A | 09/22/2004 | 17,3,19,18, 20,1 |

- ADS-40 Data Quality Control

- Prior to data delivery, the ADS-40 imagery was examined by EarthData Aviation to assure the data quality.  This quality control check involved examining raw images from every array of the ADS-40 for every flight.  These images were examined for any data anomalies that might occur such as pixel drop-outs.  In addition to this check of the raw images, the nadir image from each flightline was rectified to a plane at a subsampled pixel resolution.  These georeferenced images were imported into viewing software and then examined.  The examination of the rectified images was used to identify any clouds, cloud shadows, data gaps, smoke, etc… Any possible issues that were identified during this QC were then addressed on a case by case basis and reflights were scheduled where applicable.

GPS Processing Graphs

Forward-Reverse Solution Residuals

Lift 26404A: 20 September 2004

Aircraft Trajectory Plot

Lift 26404A: 20 September 2004

Forward-Reverse Solution Residuals

Lift 26604A: 22 September 2004

Aircraft Trajectory Plot

Lift 26604A: 22 September 2004

NGS Datasheets

National Geodetic Survey,   Retrieval Date = SEPTEMBER 17, 2004

AI5582 ***********************************************************************

AI5582  PACS        -  This is a Primary Airport Control Station.

AI5582  DESIGNATION -  HVN D

AI5582  PID         -  AI5582

AI5582  STATE/COUNTY-  CT/NEW HAVEN

AI5582  USGS QUAD   -  NEW HAVEN (1984)

AI5582

AI5582                         *CURRENT SURVEY CONTROL

AI5582  ___________________________________________________________________

AI5582* NAD 83(1996)-  41 16 04.79476(N)    072 53 18.39305(W)     ADJUSTED

AI5582* NAVD 88     -         1.33   (meters)       4.4    (feet)  GPS OBS

AI5582  ___________________________________________________________________

AI5582  X           -   1,412,610.498 (meters)                     COMP

AI5582  Y           -  -4,588,468.632 (meters)                     COMP

AI5582  Z           -   4,184,821.020 (meters)                     COMP

AI5582  LAPLACE CORR-          -2.73  (seconds)                    DEFLEC99

AI5582  ELLIP HEIGHT-         -28.56  (meters)          (06/04/02) GPS OBS

AI5582  GEOID HEIGHT-         -29.90  (meters)                     GEOID03

AI5582

AI5582  HORZ ORDER  -  B

AI5582  ELLP ORDER  -  FOURTH    CLASS I

AI5582

AI5582.This mark is at Tweed-New Haven Airport (HVN)

AI5582

AI5582.The horizontal coordinates were established by GPS observations

AI5582.and adjusted by the National Geodetic Survey in May 2000.

AI5582

AI5582.The orthometric height was determined by GPS observations and a

AI5582.high-resolution geoid model.

AI5582

AI5582.GPS derived orthometric heights for airport stations designated as

AI5582.PACS or SACS are published to 2 decimal places.  This maintains

AI5582.centimeter relative accuracy between the PACS and SACS.  It does

AI5582.not indicate centimeter accuracy relative to other marks which are

AI5582.part of the NAVD 88 network.

AI5582

AI5582.Photographs are available for this station.

AI5582

AI5582.The X, Y, and Z were computed from the position and the ellipsoidal ht.

AI5582

AI5582.The Laplace correction was computed from DEFLEC99 derived deflections.

AI5582

AI5582.The ellipsoidal height was determined by GPS observations

AI5582.and is referenced to NAD 83.

AI5582

AI5582.The geoid height was determined by GEOID03.

AI5582

AI5582;                    North         East     Units Scale Factor Converg.

AI5582;SPC CT       -   200,682.299   293,200.107   MT  0.99999383   -0 05 30.5

AI5582;UTM  18      - 4,570,658.327   676,869.238   MT  0.99998501   +1 23 35.2

AI5582

AI5582!             -  Elev Factor  x  Scale Factor =   Combined Factor

AI5582!SPC CT       -   1.00000448  x   0.99999383  =   0.99999831

AI5582!UTM  18      -   1.00000448  x   0.99998501  =   0.99998949

AI5582

AI5582                          SUPERSEDED SURVEY CONTROL

AI5582

AI5582  ELLIP H (05/22/00)  -28.55   (m)                       GP(       ) 4 1

AI5582

AI5582.Superseded values are not recommended for survey control.

AI5582.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

AI5582.See file dsdata.txt to determine how the superseded data were derived.

AI5582

AI5582_U.S. NATIONAL GRID SPATIAL ADDRESS: 18TXL7686970658(NAD 83)

AI5582_MARKER: I = METAL ROD

AI5582_SETTING: 59 = STAINLESS STEEL ROD IN SLEEVE (10 FT.+)

AI5582_STAMPING: HVN D 1999

AI5582_MARK LOGO: NGS

AI5582_PROJECTION: RECESSED 2 CENTIMETERS

AI5582_MAGNETIC: N = NO MAGNETIC MATERIAL

AI5582_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

AI5582_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

AI5582+SATELLITE: SATELLITE OBSERVATIONS - 1999

AI5582_ROD/PIPE-DEPTH: 9.4  meters

AI5582_SLEEVE-DEPTH  : 1 meters

AI5582

AI5582  HISTORY     - Date     Condition        Report By

AI5582  HISTORY     - 1999     MONUMENTED       WOOLPT

AI5582

AI5582                          STATION DESCRIPTION

AI5582

AI5582'DESCRIBED BY WOOLPERT CONSULTANTS 1999 (ARL)

AI5582'THE STATION IS LOCATED APPROXIMATELY 4.1 MI (6.6 KM) SOUTHEAST OF NEW

AI5582'HAVEN, 1.3 MI (2.1 KM) SOUTHWEST OF EAST HAVEN AND IS LOCATED AT THE

AI5582'TWEED-NEW HAVEN AIRPORT.  OWNERSHIP-- TWEED-NEW HAVEN AIRPORT, 155

AI5582'BURR STREET ADMINISTRATION BUILDING 2ND FLOOR, NEW HAVEN CT, 06512,

AI5582'C/O RICHARD LAMPORT AIRPORT MANAGER.  PHONE NUMBER (203) 466-8833, FAX

AI5582'NUMBER (203) 466-8844.  CONTACT THE AIRPORT MANAGER AT LEAST 24 HOURS

AI5582'PRIOR TO ARRANGE FOR ACCESS.

AI5582'TO REACH THE STATION FROM THE JUNCTION OF INTERSTATE 95 AND STATE

AI5582'ROUTE 337 SOUTH (BURR STREET) , PROCEED SOUTHERLY 1.35 MI (2.17 KM)

AI5582'ALONG BURR STRRET TO THE JUNCTION OF DODGE AVENUE ON THE LEFT.  TURN

AI5582'LEFT AND PROCEED EASTERLY 1.60 MI (2.57 KM) ALONG DODGE AVENUE TO THE

AI5582'JUNCTION OF THOMAS STREET.  TURN RIGHT AND PROCEED SOUTHERLY 0.2 MI

AI5582'(0.3 KM) ALONG THOMAS STREET TO A PARKING LOT ENTRANCE ON THE RIGHT.

AI5582'TURN RIGHT AND CONTINUE SOUTHERLY 0.3 MI (0.5 KM) ALONG A ROAD AT THE

AI5582'EDGE OF THE PARKING LOT TO A GATE LEADING TO THE MAINTANCE AREA.

AI5582'PROCEED THROUGH THE GATE AND ALONG THE ROAD SOUTHWESTERLY 0.1 MI (0.2

AI5582'KM) TO THE JUNCTION OF TWY CHARLIE.  TURN RIGHT AND PROCEED NORTHWEST

AI5582'0.20 MI (0.32 KM) ON TWY CHARLIE, CROSSING RWY 2-10, TO THE STATION ON

AI5582'THE RIGHT, IN THE INFIELD FORMED BY TWY CHARLIE, RWY 2-20, AND THE

AI5582'TERMINAL RAMP.  THE STATION IS ONLINE WITH THE SECOND SET OF RWY

AI5582'LIGHTS NORTH OF TWY CHARLIE.

AI5582'THE STATION IS LOCATED 67.8 M (222.4 FT) NORTH 54 DEGREES EAST OF THE

AI5582'EDGE OF PAVEMENT OF TWY CHARLIE, 61.0 M (200.1 FT) NORTH 73 DEGREES

AI5582'WEST OF THE SECOND SET OF RWY LIGHTS NORTH OF TWY CHARLIE, AND 29.3 M

AI5582'(96.1 FT) NORTH OF A SQUARE CATCH BASIN.  THE STATION IS A STAINLESS

AI5582'STEEL ROD WITH A PUNCHMARK DRIVEN TO A SLOW DRIVING RATE AT A DEPTH OF

AI5582'9.4 M, (30.8 FT) ENCASED IN A 1 M (3.3 FT) FINNED SLEEVE FILLED WITH

AI5582'NON-TOXIC GREASE, RECESSED 8 CM BENEATH A STANDARD NGS LOGO CAP

AI5582'STAMPED-- HVN D 1999-- ENCASED IN A 0.3 M (1.0 FT) DIAMETER CONCRETE

AI5582'COLLAR AND RECESSED 2 CM BELOW THE GROUND.  THIS STATION IS DESIGNATED

AI5582'AS THE PRIMARY AIRPORT CONTROL STATION.

DF4064 ***********************************************************************

DF4064  CORS        -  This is a GPS Continuously Operating Reference Station.

DF4064  DESIGNATION -  NEW YORK WAAS 1 CORS ARP

DF4064  CORS_ID     -  ZNY1

DF4064  PID         -  DF4064

DF4064  STATE/COUNTY-  NY/SUFFOLK

DF4064  USGS QUAD   -  PATCHOGUE (1967)

DF4064

DF4064                         *CURRENT SURVEY CONTROL

DF4064  ___________________________________________________________________

DF4064* NAD 83(CORS)-  40 47 03.55019(N)    073 05 49.78137(W)     ADJUSTED

DF4064* NAVD 88     -

DF4064  ___________________________________________________________________

DF4064  EPOCH DATE  -        2002.00

DF4064  X           -   1,406,145.344 (meters)                     COMP

DF4064  Y           -  -4,627,345.099 (meters)                     COMP

DF4064  Z           -   4,144,321.822 (meters)                     COMP

DF4064  ELLIP HEIGHT-           7.26  (meters)          (02/??/03) GPS OBS

DF4064  GEOID HEIGHT-         -31.09  (meters)                     GEOID03

DF4064

DF4064  HORZ ORDER  -  SPECIAL (CORS)

DF4064  ELLP ORDER  -  SPECIAL (CORS)

DF4064

DF4064.ITRF positions are available for this station.

DF4064.The coordinates were established by GPS observations

DF4064.and adjusted by the National Geodetic Survey in February 2003.

DF4064.The coordinates are valid at the epoch date displayed above.

DF4064.The epoch date for horizontal control is a decimal equivalence

DF4064.of Year/Month/Day.

DF4064

DF4064

DF4064.The PID for the CORS L1 Phase Center is DF8964.

DF4064

DF4064.The XYZ, and position/ellipsoidal ht. are equivalent.

DF4064

DF4064.The ellipsoidal height was determined by GPS observations

DF4064.and is referenced to NAD 83.

DF4064

DF4064.The geoid height was determined by GEOID03.

DF4064

DF4064;                    North         East     Units Scale Factor Converg.

DF4064;SPC NY L     -    68,980.664   376,205.870   MT  0.99999556   +0 35 25.9

DF4064

DF4064!             -  Elev Factor  x  Scale Factor =   Combined Factor

DF4064!SPC NY L     -   0.99999886  x   0.99999556  =   0.99999442

DF4064

DF4064                          SUPERSEDED SURVEY CONTROL

DF4064

DF4064.No superseded survey control is available for this station.

DF4064

DF4064_U.S. NATIONAL GRID SPATIAL ADDRESS: 18TXL6055716557(NAD 83)

DF4064_MARKER: STATION IS THE ANTENNA REFERENCE POINT OF THE GPS ANTENNA

DF4064

DF4064                          STATION DESCRIPTION

DF4064

DF4064'DESCRIBED BY NATIONAL GEODETIC SURVEY 2003

DF4064'STATION IS A GPS CORS.  LATEST INFORMATION INCLUDING POSITIONS AND

DF4064'VELOCITIES ARE AVAILABLE IN THE COORDINATE AND LOG FILES ACCESSIBLE

DF4064'BY ANONYMOUS FTP OR THE WORLDWIDE WEB.

DF4064'   FTP CORS.NGS.NOAA.GOV: CORS/COORD  AND  CORS/STATION_LOG

DF4064'   HTTP://WWW.NGS.NOAA.GOV  UNDER PRODUCTS AND SERVICES.

DF8964 ***********************************************************************

DF8964  CORS        -  This is a GPS Continuously Operating Reference Station.

DF8964  DESIGNATION -  NEW YORK WAAS 1 CORS L1 PHASE CENTER

DF8964  CORS_ID     -  ZNY1

DF8964  PID         -  DF8964

DF8964  STATE/COUNTY-  NY/SUFFOLK

DF8964  USGS QUAD   -  PATCHOGUE (1967)

DF8964

DF8964                         *CURRENT SURVEY CONTROL

DF8964  ___________________________________________________________________

DF8964* NAD 83(CORS)-  40 47 03.55012(N)    073 05 49.78136(W)     ADJUSTED

DF8964* NAVD 88     -

DF8964  ___________________________________________________________________

DF8964  EPOCH DATE  -        2002.00

DF8964  X           -   1,406,145.430 (meters)                     COMP

DF8964  Y           -  -4,627,345.382 (meters)                     COMP

DF8964  Z           -   4,144,322.076 (meters)                     COMP

DF8964  ELLIP HEIGHT-           7.65  (meters)          (02/??/03) GPS OBS

DF8964  GEOID HEIGHT-         -31.09  (meters)                     GEOID03

DF8964

DF8964  HORZ ORDER  -  SPECIAL (CORS)

DF8964  ELLP ORDER  -  SPECIAL (CORS)

DF8964

DF8964.ITRF positions are available for this station.

DF8964.The coordinates were established by GPS observations

DF8964.and adjusted by the National Geodetic Survey in February 2003.

DF8964.The coordinates are valid at the epoch date displayed above.

DF8964.The epoch date for horizontal control is a decimal equivalence

DF8964.of Year/Month/Day.

DF8964

DF8964

DF8964.The PID for the CORS ARP is DF4064.

DF8964

DF8964.The XYZ, and position/ellipsoidal ht. are equivalent.

DF8964

DF8964.The ellipsoidal height was determined by GPS observations

DF8964.and is referenced to NAD 83.

DF8964

DF8964.The geoid height was determined by GEOID03.

DF8964

DF8964;                    North         East     Units Scale Factor Converg.

DF8964;SPC NY L     -    68,980.662   376,205.870   MT  0.99999556   +0 35 25.9

DF8964

DF8964!             -  Elev Factor  x  Scale Factor =   Combined Factor

DF8964!SPC NY L     -   0.99999880  x   0.99999556  =   0.99999436

DF8964

DF8964                          SUPERSEDED SURVEY CONTROL

DF8964

DF8964.No superseded survey control is available for this station.

DF8964

DF8964_U.S. NATIONAL GRID SPATIAL ADDRESS: 18TXL6055716557(NAD 83)

DF8964_MARKER: STATION IS THE L1 PHASE CENTER OF THE GPS ANTENNA

DF8964

DF8964                          STATION DESCRIPTION

DF8964

DF8964'DESCRIBED BY NATIONAL GEODETIC SURVEY

DF8964'STATION IS A GPS CORS.  LATEST INFORMATION INCLUDING POSITIONS AND

DF8964'VELOCITIES ARE AVAILABLE IN THE COORDINATE AND LOG FILES ACCESSIBLE

DF8964'BY ANONYMOUS FTP OR THE WORLDWIDE WEB.

DF8964'   FTP CORS.NGS.NOAA.GOV: CORS/COORD  AND  CORS/STATION_LOG

DF8964'   HTTP://WWW.NGS.NOAA.GOV  UNDER PRODUCTS AND SERVICES.